SECOM Test Project

Test report

Draft version

Content

1	SECOM Test Project	6

1.1	Purpose and expected outcome	6

1.2	General	6

1.3	Examples of questions	7

1.4	Overview of Test Objectives	8

1.5	Overview of Test Cases	9

2	Observations	10

3	Testbed	13

3.1	General	13

3.2	Testbed A	13

3.2.1	Description	13

3.2.2	Functionality in the testbed	14

3.3	Testbed B	15

3.3.1	Description	15

3.3.2	Functionality in the testbed	15

3.3.3	Requirements and input	15

3.4	Testbed C	16

3.4.1	Description	16

3.4.2	Functionality in the testbed	16

4	Test Case 1 - Data protection (signing) of unclassified data	18

4.1	Description	18

4.1.1	Test objectives	18

4.1.2	Acceptance Criteria	18

4.1.3	Test Scenarios	18

4.1.4	Test Environment	18

4.1.5	Test tools	18

4.1.6	Test data	18

4.1.7	Test procedure	18

4.2	Test results and discussions	20

4.3	Conclusions and Recommendations	21

5	Test Case 2 - Data protection of classified data (signing and encryption)	22

5.1	Description	22

5.1.1	Test objectives	22

5.1.2	Acceptance Criteria	22

5.1.3	Test Scenarios	22

5.1.4	Test Environment	22

5.1.5	Test tools	22

5.1.6	Test data	22

5.1.7	Test procedure	22

5.2	Test results and discussions	23

5.3	Conclusions and Recommendations	23

6	Test Case 3 – SECOM PKI	24

6.1	Description	24

6.1.1	Test objectives	24

6.1.2	Acceptance Criteria	24

6.1.3	Test Scenarios	24

6.1.4	Test Environment	24

6.1.5	Test tools	24

6.1.6	Test data	24

6.1.7	Test procedure	24

6.2	Test results and discussions	25

6.3	Conclusions and Recommendations	25

7	Test Case 4 – Exchange large data	26

7.1	Description	26

7.2	Targeted questions	26

7.3	Test Functionality	26

7.4	Test Variables	26

7.5	Testbed	26

7.6	Test Sequence	26

7.7	Test results and discussions	29

7.8	Conclusions and Recommendations	29

8	Test Case 5 – Exchange compressed data	29

8.1	Description	29

8.1.1	Test objectives	29

8.1.2	Acceptance Criteria	29

8.1.3	Test Scenarios	29

8.1.4	Test Environment	29

8.1.5	Test tools	30

8.1.6	Test data	30

8.1.7	Test procedure	30

8.2	Test results and discussions	31

8.3	Conclusions and Recommendations	31

9	Test Case 6 – Closed loop communication	32

9.1	Description	32

9.1.1	Test objectives	32

9.1.2	Acceptance Criteria	32

9.1.3	Test Scenarios	32

9.1.4	Test Environment	32

9.1.5	Test tools	32

9.1.6	Test data	32

9.1.7	Test procedure	32

9.2	Test results and discussions	33

9.3	Conclusions and Recommendations	33

10	Test Case 7 – Subscribe to data	34

10.1	Description	34

10.1.1	Test objectives	34

10.1.2	Acceptance Criteria	34

10.1.3	Test Scenarios	34

10.1.4	Test Environment	34

10.1.5	Test tools	34

10.1.6	Test data	34

10.1.7	Test procedure	34

10.2	Test results and discussions	35

10.3	Conclusions and Recommendations	35

11	Test Case 8 – Service information	36

11.1	Description	36

11.1.1	Test objectives	36

11.1.2	Acceptance Criteria	36

11.1.3	Test Scenarios	36

11.1.4	Test Environment	36

11.1.5	Test tools	36

11.1.6	Test data	36

11.1.7	Test procedure	36

11.2	Test results and discussions	37

11.3	Conclusions and Recommendations	37

12	Test Case 9 – Service status	38

12.1	Description	38

12.1.1	Test objectives	38

12.1.2	Acceptance Criteria	38

12.1.3	Test Scenarios	38

12.1.4	Test Environment	38

12.1.5	Test tools	38

12.1.6	Test data	38

12.1.7	Test procedure	38

12.2	Test results and discussions	39

12.3	Conclusions and Recommendations	39

13	Test Case 10 - Cyber Security Review	40

13.1	Description	40

13.2	Targeted questions	40

13.3	Test Functionality	40

13.4	Test Variables	41

13.5	Testbed	41

13.6	Test Sequence	41

13.7	Test results and discussions	41

13.7.1	Observations	41

13.8	Conclusions and Recommendations	41

14	Test Case 11 – White list and access request	42

14.1	Description	42

14.1.1	Test objectives	42

14.1.2	Acceptance Criteria	42

14.1.3	Test Scenarios	42

14.1.4	Test Environment	42

14.1.5	Test tools	42

14.1.6	Test data	42

14.1.7	Test procedure	42

14.2	Test results and discussions	43

14.3	Conclusions and Recommendations	43

15	Test Case 12 – Service Discovery	43

15.1	Description	43

15.2	Targeted questions	44

15.3	Test Functionality	44

15.4	Test Variables	44

15.5	Testbed	44

15.6	Test Sequence	46

15.7	Test results and discussions	46

15.8	Conclusions and Recommendations	46

16	Test Case 13 – Exchange of several different payloads	46

16.1	Description	46

16.1.1	Test objectives	46

16.1.2	Acceptance Criteria	46

16.1.3	Test Scenarios	46

16.1.4	Test Environment	46

16.1.5	Test tools	47

16.1.6	Test data	47

16.1.7	Test procedure	47

16.2	Test results and discussions	48

16.3	Conclusions and Recommendations	48

17	ANNEX A Participant (SMA)	49

17.1	Planning	49

17.1.1	Schedule	49

17.1.2	Input	49

17.2	APPENDIX SECOM Test Project SMA Report	50

18	ANNEX B Participant (Saab)	50

18.1	Planning	50

18.1.1	Schedule	50

18.1.2	Input	50

18.2	APPENDIX SECOM Test Project SAAB Report	51

19	ANNEX C Participant (StormGeo)	52

19.1	Planning	52

19.1.1	Schedule	52

19.1.2	Input	52

19.2	APPENDIX SECOM Test Project StormGeo Report	52

SECOM Test Project

Purpose and expected outcome

The purpose is to challenge the IEC 63173-2 SECOM standard draft and provide feedback to WG17. The purpose is also to get more experience and example data of actually implementing the standard.

The expected outcomes from the project are

- feedback with recommendations to WG17 for improvement and stabilizing the SECOM standard.  Completeness, Correctness, Consistency

- demonstrator, but not the primary goal

- example data and example commands, (tbd which example data that is of later use)

- bonus if the outcome is data and even a “simulator” or “reference service” that can support “Test methods and expected result” clauses and how a party can test if their equipment is compliant with SECOM

General

SECOM standard is expected to be as standalone as possible, yet aligned with other standards. And standalone means here that a user shall be able to implement secure exchange and communication of S-100 based products based on the IEC 63173-2 SECOM standard document.

SECOM specifies service interfaces (APIs) for data exchange, data protection measures to enable secure communication and interfaces for service discoverability. SECOM is applicable for S-100 based products but also other data (payload) formats are supported i.e. SECOM is generally independent of which data type is exchanged.

SECOM contains several parts or components that are needed to achieve secure exchange, but different parties will implement different parts/components into their product, and by nature an exchange involves at least two parties where one is sending and one is receiving, and in many cases the roles changes during an interaction and most parties will most likely need to implement the SECOM as both sender and receiver (provider and consumer of information).

Description of below figure…TBD

Examples of questions

- How does the signing request to SECOM PKI work?

- What are the SECOM requirements for the asymmetric keys in PKI?

- What are the SECOM requirements for the encryption key

- How exchange the encryption key?

- How use keys from e.g. PRIMAR with SECOM solution?

- How relate keys for signing data to keys for TLS encryption? E.g. for a ship, for a VTS.

- Shall SECOM Communication Channel Security and service authentication be based on TLS and X.509 Certificates or OpenID, HMAC or other standard using the HTTP Headers instead?

- What are the requirements, and are the met, on SECOM Service Discovery Interface?

Overview of Test Objectives

The following Test Objectives have been identified as candidates for SECOM Test Project.

| Test Objective | Description/ rationale | SECOM solution | Test case(s) |
| --- | --- | --- | --- |
| - Message integrity | Ensures the complete message is unchanged between SECOM services | Envelope signing | 1 |
| - Data integrity* | Data integrity ensures the data transmitted is accurate and consistent | Data signing | 1 |
| - Transport confidentiality | Ensure communication channel protection between SECOM services | Channel encryption (TLS) | 1, 2 |
| - Data protection | Data protection ensures information is confidential except for the intended recipient | Data encryption (AES) | 2 |
| - Service identity | Support SECOM service identification | Service certificate authentication (X.509 PKI) | 1, 3 |
| - Client identity* | Support client identification | End user client certificate authentication (X.509 PKI) | 1, 3 |
| - Client authorization | Facilitate client access to information | Service authorization | 11 |
| - Bandwidth optimization | Minimize size of data package sent to reduce required bandwidth | Data compression (GZIP, Deflate) | 5 |
| - Large message transfer | Facilitate large message transfer i.e. message sizes > 350 kB | Link to data facilitated by interfaces Upload Link, Get By Link | 4 |
| - Closed loop communication | Notification of message received/ read etc. to ensure dialogue between end-user applications. | Acknowledgement message (acknowledgement interface) | 6 |
| - Service discoverability | Search for services by means of service metadata. In order to locate relevant services for consumption. | Service registry lookup (Search Service interface) | 12 |
| - Information push | Share information by uploading data to a service | Service interface to receive uploaded information (Upload interface) | 2 |
| - Information pull | Retrieve information by downloading data from a service | Service interface to retrieve information (Get interface) | 1 |
| - Subscribe to data | Subscribe to information to receive subsequent updates | Service interfaces for Subscription request, remove & notify | 7 |
| - Service information | To facilitate information wrt service accepted payloads and endpoints | Service capability interface | 8 |
| - Service condition | Contextual service status to check service operation. | Ping interface for checking last interactionTime with end user application and SECOM service status | 9 |
| - Payload agnostic service | Caters for exchanging payloads of different types | To be investigated | 13 |

* Data integrity and client identity test objectives are only relevant if SECOM PKI is used for end user client certificates.

Overview of Test Cases

The following Test Cases have been identified to cover previously described test objectives.

| Test Case | Name | Interface | Document Reference | Test objects | Testbed target |
| --- | --- | --- | --- | --- | --- |
| Test Case 1 | Data protection of unclassified data (signing) | Upload | 5.7.2, 6, 7.3 | 1, 2, 3, 5, 6, 13 | Testbed B |
| Test Case 2 | Data protection of classified data (signing and encryption) | Get, EncryptionKey | 5.7.2, 6, 7.3, 7.4, 7.5 | 1, 2, 3, 4, 5, 6, 12 | Testbed B |
| Test Case 3 | SECOM PKI |  | 8 |  | Testbed B |
| Test Case 4 | Exchange Large Data | Upload Link, Get By Link | 5.6.4 + 5.6.8 | 9 | Testbed B |
| Test Case 5 | Exchange compressed data | Upload | 7.2 | 8 | Testbed B |
| Test Case 6 | Closed loop communication | Acknowledgement | 5.7.4 | 10 | Testbed B |
| Test Case 7 | Subscribe to data | Subscription request, remove & notify | 5.7.10, 5.7.11, 5.7.12 | 14 | Testbed B |
| Test Case 8 | Service information | Capability | 5.7.13 | 15 | Testbed B |
| Test Case 9 | Service status | Ping | 5.7.14 | 16 | Testbed B |
| Test Case 10 | Cyber Security Review |  | 4 + 6 + 7 + (5) |  | Testbed B |
| Test Case 11 | White list and access request |  | 5 + Annex D/E | 7 | Testbed B |
| Test Case 12 | Service Discovery | Search Service | 9 | 11 | Testbed B |
| Test Case 13 | Exchange several different types of payloads | Capability, Upload, Get | ? | 17 | Testbed C |

Observations

| Num | Ref number | Observation | Found when … | Consequence/Proposal | Reference in SECOM Document |
| --- | --- | --- | --- | --- | --- |
| 001 | SMA-001 STT-001 | Converting signature to HEX may add carriage return signs, but it can also be on one line Today not described in SECOM | Building Testbed A | - affects the JSON or XML - affects interoperability Need to be described in SECOM. | 7.3 Data Protection |
| 002 | STT-002 | Data may need padding if shorter than signing key Today not described in SECOM |  |  | 7.3 Data Protection |
| 003 | SMA-002 | SECOM v20xx is missing description of what to sign. Q: What if the data is compressed? converted to Base64? Encrypted? Shall the original data file always be signed? Or shall the file prepared for transfer be signed? | Building Testbed A | The different alternatives is elaborated in 4.7.1 Discussion: What shall be signed? | 7.3 Data Protection 5 Information Service Interface |
| 004 | SMA-003 | Update Link and Get By Link is not well defined in the document | Building Testbed B and the swagger file | Impact on the swagger file | Interface Upload Link and Get By Link |
| 005 | SMA-004 | It may be a security risk to exchange a URL to any external storage/web page | Building Testbed B and the swagger file | One approach could be that instead of uploading a URL, an identifier is uploaded, and then Get can be used to retrieve the object attached to the identifier. | Interface Upload Link and Get By Link |
|  | SMA-006 | Should both ResponseObject and Error ResponseObject be defined for every REST interface? | Building Testbed B and the swagger file | Removed when moved to Specification part. But it may need to be defined for the REST design of the interface. |  |
|  | SMA-007 | Many identifiers in the different service interfaces makes it messy. | Building Testbed B and the swagger file | Propose to clean up and make consistent, and remove unnecessary identifiers. |  |
|  | SMA-008 | What shall the REST operation be named for Upload and Get now when we removed the type of message in the URL? Object, Message | Building Testbed B and the swagger file |  |  |
|  | SMA-009 | When data is incorporated in JSON object, such as in UploadObject, it tricky to restore the data exactly to match the original, hence difficult to verify the signature. |  |  |  |
|  | SMA-010 | Is it necessary/beneficial to exchange a filename as well? If the data is compressed into a ZIP-file you need to open and see the files inside before handle it further. |  |  |  |
|  | SMA-011 | Currently the UploadObject contains a SECOM_SecurityMetadata object based on S-100. Another alternative would be to always exchange an ExchangeSet ZIP containing the necessary metadata to verify the signature, filename etc. |  | This requires however a stable S-100 to finalize SECOM. The metadata is always used in GET and ACK. |  |

Testbed

General

Three testbeds, called Testbed A, Testbed B and Testbed C, have been defined for this project. The functionality and connectivity is increased in each testbed.

The first step (Testbed A) is local implementation and email for connectivity.

Second step (Testbed B) is still local implementations with local services as intermediate step. Perhaps this testbed will be the closest design for the Test methods described in SECOM.

Third step (Testbed C) is local implementation connected through services and internet with adequate protection according to SECOM.

Testbed A

Description

The purpose with testbed A is to test SECOM Data Protection Scheme.

PKI in the testbed

The approach is to use MCP and Navelink as far as possible as PKI system. Where SECOM goes beyond MCP/Navelink, manual commands using e.g. openssl need to be used.

- SMA creates a Service ID called “SECOM SMA Test Service ID” and issues certificate for that entity. The public certificate (Download the public part) is then emailed to Saab.

- Saab creates a Service called SECOM-SAAB and issues certificate for that entity. The public key/certificate is then emailed to SMA.

MCP/Navelink cannot today provide all public keys to other than user within same organization, hence public keys between organizations need today be mail around.

Functionality in the testbed

The following functionality is required in the testbed:

- Create Signature on a payload

  - Calculate checksum according to SECOM (S-100)

  - Encrypt checksum with own private key

- Exchange payload and SECOM_ExchangeMetadata objects

- Verify signature

  - Retrieve public keys for claimed identity

- Encrypt payload

  - Generate secret key

  - (compress data)

  - Encrypt data

- Compress data

- Exchange secret key

  - Encrypt secret key (RSA or ECC, diffie-hellman or similar)

  - Sign secret key

- Decrypt payload

  - Receive secret key

  - Verify signature

  - Decrypt secret key

  - Decrypt data

  - (uncompress data)

- Uncompress data

Testbed B

Description

The purpose with Testbed B is to test SECOM Data Protection in combination with SECOM transport security and SECOM service interface within each partners own environment.

Testbed B is an intermediate test bed before achieving Testbed C, but also a “plan B” if firewalls and connection between partners for some reason fails.

Functionality in the testbed

See Testbed C for list of functionality.

This testbed might be the testbed closest to what can/will be used in Test methods in SECOM.

Requirements and input

- IEC 63173-2 SECOM

  - Annex X: OpenAPI (swagger)

Testbed C

Description

The purpose with Testbed C is to test SECOM Data Protection in combination with SECOM transport security and SECOM service interface and connections across different organizations networks.

Functionality in the testbed

The following functionality is required in the testbed:

- Create Signature on a payload

  - Calculate checksum according to SECOM (S-100)

  - Encrypt checksum with own private key

- Exchange payload and SECOM_ExchangeMetadata objects

- Verify signature

  - Retrieve public keys for claimed identity

- Encrypt payload

  - Generate secret key

  - (compress data)

  - Encrypt data

- Compress data

- Exchange secret key

  - Encrypt secret key (RSA or ECC, diffie-hellman or similar)

  - Sign secret key

- Decrypt payload

  - Receive secret key

  - Verify signature

  - Decrypt secret key

  - Decrypt data

  - (uncompress data)

- Uncompress data

- Expose (provide) deployed and consumable service interface 
OBS! Discuss which interfaces that need logic.

  - Upload

  - Upload Link

  - Get Summary

  - Get

  - Get by Link

  - Acknowledgement

  - Subscribe

  - Remove subscription

  - Subscription notofication

  - Request Access

  - Access Notification

  - Capability

  - Ping

  - EncryptionKey

- Consume service interface

- Transport Security and Service Authentication

  - Encrypt traffic

  - Authenticate client in service call

  - Decrypt traffic

- Send Sign request

  - Create signing request

  - Call SECOM PKI Signing Request (secure, authentication)

- Search for service to consume

- SECOM PKI: Provide Public Keys

- SECOM PKI: Handle Signing request

  - Authentication

  - Handle Signing request and store Public Key

- SECOM Service Registry: Handle search request for service

Test Case 1 - Data protection (signing) of unclassified data

Description

The test case focus on exchange of unclassified signed data, and the verification and authentication of the signature.

Test objectives

- Message integrity

- Data integrity

- Transport confidentiality

- Service identity

- Information push

Acceptance Criteria

Message integrity verified by comparing calculated envelope signature with the corresponding received signature.

Data integrity can be validated providing the end-user application public certificate is issued by SECOM PKI. In other cases the data integrity has to be validated in the end-user application which out of scope for SECOM.

Transport confidentiality verified by establishing an encrypted channel using SECOM PKI issued SSL host certificates.

Service identity verified against SECOM PKI using provided “client” certificate received in TLS session.

Information push achieved by a successful data uploaded.

Test Scenarios

Actor A shall send one data object (XML) to Actor B. Actor A decides that the information is unclassified. The data is encapsulated into an UploadObject ready to be exchanged as body to a SECOM Upload Service Interface.

Test Environment

Testbed B

Test tools

OpenSSL, Notepad++

Test data

Steps to prepare unclassified and uncompressed data.

The data in this example is one RTZ.

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| Select data file |  | data-1.rtz |
| Convert to Base64 | openssl base64 -A -in data-1.rtz -out data-1.rtz.base64 | data-1.rtz.base64 |
| Select Private Key |  | PrivateKey_SECOM_SMA_Test_Service_ID.pem |
| Create signature using original data file (data-1.rtz) | openssl dgst -sha256 -sign PrivateKey_SECOM_SMA_Test_Service_ID.pem data-1.rtz > data-1.rtz.sig | data-1.rtz.sig |
| Convert signature to HEX | xxd -u -ps -c 120 data-1.rtz.sig > data-1.rtz.sig.hex | data-1.rtz.sig.hex |
| Select Public Certificate for the data object |  | Certificate_SECOM_SMA_Test_Service_ID.pem |
| Select envelope public certificate |  | EnvelopeCertificate.pem |
| Set other metadata values for the Envelope Object |  | Envelope.json |
| Sign envelope |  | Envelope.sig |
| Add envelope signature to upload object |  |  |
| Create Upload Object in JSON |  | UploadObject-1.json |
| SECOM A |  |  |
| Add client certificate Actor A |  |  |
| Verify receiver host certificate SECOM B | Check certificate against SECOM Pki | True/false |
| Create TLS |  | Encrypted channel established |
| Consume Actor B SECOM Api Upload | POST URL/v1/object?parameters {body} : return |  |

RECEIVER

Verify Signature(s) and restore data

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Receive … |  | UploadObject deserialized |
| Authenticate sender | Verify client cert for Actor A from TLS against SECOM Pki | True/false |
| Parse envelope |  | uploadObject.Envelope |
| Convert to byte[] |  | Envelope[] |
| Authenticate data and claimed public key | Verify envelope signature | True/false |
| Store(inform Actor B OEM) incoming object |  | UploadObject-1.json |
| Actor B |  |  |
| Parse and store data from object |  | data.rtz.base64 |
| Parse and store signature from object |  | data.sig.hex |
| Restore signature from HEX | xxd -r -u -ps data.sig.hex > data.sig | data.sig |
| Parse and store public certificate from object |  | publicCert.pem |
| Identify Root Certificate through the Thumbprint |  | mc-ca-chain_staging.pem |
| Verify Certificate | openssl verify -CAfile mc-ca-chain_staging.pem publicCert.pem > publicCert.pem.verification | publicCert.pem.verification |
| Extract public keys from certificate | openssl x509 -in publicCert.pem" -pubkey -noout > publicCert_key.pem" | publicCert_key.pem |
| Restore data from Base64 to original | openssl base64 –A -d -in data.rtz.base64 -out data.rtz | data.rtz |
| Verify signature i.e. compare signature with original data file | openssl dgst -sha256 -verify publicCert_key.pem -keyform pem -signature data.sig data.rtz > data.sig.verification | Verified OK |
| Compare received data with restored data | Open received data.rtz and restored data.rtz (Notepad++ Compare Plugin) | Received data unchanged |

Test results and discussions

This chapter contains the common results and discussions around the test case.

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |
| Set other metadata values for the Envelope Object | Attribute data type values might differ in various OS/ languages. | There is a need to agree on attribute data types used in SECOM to ensure interoperability. | Introduce S100 basic data types to have a common description. |
| Sign envelope | Signature not consistent in different implementations | Order for attributes important when creating the EnvelopeObject to be signed. | Added an ordering column in the EnvelopeObject |
| Create Upload Object in JSON | Conversion of keys to JSON format is implementation specific. | The conversion of keys to JSON need to be agreed upon. | Minify keys to create a uniform way of sending keys in JSON format. To secure interoperability in different OS. |
| Receive … | Deserialization issue, not consistent between different implementations | Attribute data type issue and ordering important to be able to verify signature at the receiving end. | Added an ordering column in the EnvelopeObject |

Conclusions and Recommendations

| Acceptance criteria | Solution | Conclusions | Recommendations |
| --- | --- | --- | --- |
| Message integrity | Envelope signature creation and verification | Depends on OS and implementation for consistency between services. | Introduce envelope metadata attribute ordering, commonly defined data types and rules for serialization of JSON uploadObject. |
| Data integrity | Data signature creation and verification | If created in SECOM PKI this can be validated in the SECOM service instance. | Introduce ordering for attributes in the DigitalSignatureValueObject. |
| Transport confidentiality | TLS using PKI issued SSL certificates | Can be an issue if SSL certificates are self-signed, hence not possible to use in certain implementations (Azure). | Suggest to use official CA issued certificates. |
| Service identity | Received service instance “client” certificate | Mutual authentication on service instance level by using exchanged SECOM PKI certificates. | Important to agree beforehand on entities in certificate information for “Subject distinguished name” for authentication purposes. |
| Information push | Upload interface | Response code if total base 64 encoded message exceeds web server maximum message size missing. | Introduce response code 413 / message if uploaded message > maximum message size (350 kB). |

Test Case 2 - Data protection of classified data (signing and encryption)

Description

The test case focus on exchange of classified signed data, and the verification and authentication of the signature.

Publish and get encrypted data

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 3 – SECOM PKI

Description

The test case focus on exchange of classified signed data, and the verification and authentication of the signature.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 4 – Exchange large data

Description

- What to sign?

- Different protection schemes for each layer

- Sign ZIP file with signed data files

Targeted questions

Test Functionality

Test Variables

The following test variables in this test case have been identified:

- Data type and packaging

- Packaging of exchange metadata

Testbed

Test Sequence

- Description: Test exchange of large data e.g. grib data and ENC encrypted with S-63 procedure (Primar) (SECOM task 2020-03)

- Purpose:

Verify that SECOM interface is suitable for large data

Use Case

Actor 1 wants to send large data block (>10MB) to Actor 2.

Actor 1 creates temporary storage, loads it with data and retrieves a link to it

Actor 1 sends (uploads) the link to Actor2

Actor 2 retrieves the data using the link

Actor 1 deletes the data after X minutes/hours

Discussions

Q: What is the Temporary Storage?

FTP?

Webserver with file?

Q: What security is required?

Open? (and refer to Data Encryption if classified data?)

FTP login?

Certificates from SECOM PKI?

OpenID/token from SECOM PKI?

Sequence - Exchange large data

Last modified 2020-04-14

Sequence - Exchange large data

Sequence diagram for Upload Link to classified data

Test results and discussions

This chapter contains the common results and discussions around the test case. For each individual participants result see the Annexes in end of this document.

Conclusions and Recommendations

Missing HTTP response code added

Filter attribute added

Test Case 5 – Exchange compressed data

Description

The test case focus on exchange of compressed data.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 6 – Closed loop communication

Description

The test case focus on notification of messages received/ read etc.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 7 – Subscribe to data

Description

The test case focuses on subscription to information for receiving subsequent updates of information.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 8 – Service information

Description

The test case focus on information with regards to service information i.e. accepted payloads, endpoints etc.

Test cases for; upload, uploadLink, get, getByLink, getSummary, subscription, access and encryptionKey dependant on provided payload and version.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 9 – Service status

Description

The test case focus on retrieving the contextual status of a service.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

Test Case 10 - Cyber Security Review

Description

This test case is performed as a review of the SECOM document. If possible, the changes and variables made and recommended during the SECOM Test Project is also reviewed.

Targeted questions

- Review of the SECOM security solution as a whole

  - Overall description (Clause 4) of the SECOM standard

  - Overall solution of SECOM Data Protection (Clause 7)

  - Overall solution of SECOM Transport Security (Clause 6)

  - Data protection and Transport security applied on/in SECOM Information Service (REST)

- Review of specific issues

  - How exchange the secret key if SECOM Data Protection scheme is used instead of IHO and Permits?

  - What requirements does the exchange of the secret key put on the algorithm used in asymmetric keys used mainly for signing data? Will it work the same with ECC keys as for RSA keys? Or are there different procedures required for the encryption of the secret key?

  - Shall SECOM handle Nonrepudiation?

  - Is there a security risk with the Acknowledgement procedure described?

  - Exchange the digital signature in service defined attribute, PERMIT.xml, S100_DatasetDiscoveryMetadata,  S100_CatalogueMetadata or in SECOM_ServiceExchangeMetadata?

  - How shall the signature be transferred?

  - Which SECOM service interfaces requires signature on the payload (all interfaces that includes payload…)?

  - How ensure authentication in those interfaces that do not exchange payload and its signature?

  - Shall SECOM describe which identity to use for signing data versus service authentication (signing transport?

  - Should SECOM support session based interaction a'la S-100/Offis examples, or MMS style in MCP?

  - Shall SECOM describe Service Authentication as normative?

  - Shall SECOM Communication Channel Security be based on TLS and Certificates or OpenID/HMAC or similar?

  - Is it/will it be accepted to mandate the use of Client Certificates from SECOM PKI?

  - Should/Shall SECOM also mandate use of host certificate from SECOM PKI? Which then becomes a selfsigned certificate? Will it work in reality?

Test Functionality

N/A

Test Variables

N/A

Testbed

N/A

Test Sequence

N/A

Test results and discussions

This chapter contains the common results and discussions around the test case. For each individual participants result see the Annexes in end of this document.

Observations

| Num | Ref number | Observation | Consequence/Proposal | Reference in SECOM Document |
| --- | --- | --- | --- | --- |
| 001 |  |  |  |  |

Conclusions and Recommendations

Test Case 11 – White list and access request

Description

The test case focus on client access and client identity together with functions for white listing services.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

Fine granular organizationstructure, access on organizational level

Servicecertificate as a basis for access control which means attributes in the certificate would limit the granularity possible for access

Test Case 12 – Service Discovery

Description

The test case focus on service instance discovery.

The main purpose is to challenge

- 9 SECOM Service Discoverability.

  - Service Discovery Interface definition file (Swagger)

Targeted questions

- Fixed search parameter in interface, or dynamic query with key:value pair with recommended list of search parameters.

Test Functionality

Test Variables

Testbed

Example 1: geometry combined with serviceType search

REQUEST

Search for services with provided geometry inside service coverage area and service type “Port Call Synchronization”.

Figur 1 - Request find service with geometry and query

- https://serviceregistry.navelink.org/api/_searchGeometryWKT/serviceInstance?geometry=LINESTRING(17.39%2060.70%2C%2020.41%2059.80%2C%2017.25%2056.43)&includeDoc=false&includeNonCompliant=false&query=serviceType%3A%20Port%20Call%20Synchronization

RESPONSE

Figur 2 - Response from service registry

Example 2: Search with AND/ OR condition

REQUEST

Search for services with specific IMO and MMSI OR services with name containing “Baltic”.

Query = (imo: 9443255 AND mmsi: 276779000) OR name: Baltic

https://serviceregistry.navelink.org/api/_search/serviceInstance?includeDoc=false&includeNonCompliant=false&query=(imo%3A%209443255%20AND%20mmsi%3A%20276779000)%20OR%20name%3A%20Baltic

RESPONSE

Figur 3 - Response from service registry

Test Sequence

Test results and discussions

This chapter contains the common results and discussions around the test case. For each individual participants result see the Annexes in end of this document.

Conclusions and Recommendations

Test Case 13 – Exchange of several different payloads

Description

The test case focus on exchanging payloads of different types.

Test objectives

TBD

Acceptance Criteria

TBD

Test Scenarios

TBD

Test Environment

TBD

Test tools

TBD

Test data

TBD

Test procedure

SENDER

| Step | Commands | Result |
| --- | --- | --- |
| Actor A |  |  |
| SECOM A |  |  |

RECEIVER

| Step | Commands | Result |
| --- | --- | --- |
| SECOM B |  |  |
| Actor B |  |  |

Test results and discussions

This chapter contains the common results and discussions around the test case.

TBD

| Step | Test comments | Conclusions | Outcome |
| --- | --- | --- | --- |

Conclusions and Recommendations

TBD

ANNEX A Participant (SMA)

Planning

- (Mikael) Implement Testbed A

  - Implement exchange of unclassified signed data (messages such as RTZ, S-421, S-124)

  - Implement exchange of classified signed data (messages such as RTZ, S-421, S-124)

  - Implement exchange of large classified data (big data as GRIP data, ENC data)

- (Mattias) Implement Testbed B with functionality as Ship and service provider

  - Implement core of Testbed A

  - Implement exchange of large classified data (big data as GRIP data, ENC data)

  - Implement SECOM Information Service Interface

    - Upload

    - tbd

  - Implement SECOM Data Protection

- Implement Testbed C

- Figure out how SECOM PKI and SECOM Discovery can be realized using MCP or Navelink

Schedule

See PP for detailed schedule

  - Meetings

  - Work

  - Vacation

  - Milestones

Input

Input will be located on ProjectPlan (https://service.projectplace.com/)

- IEC 63173-2 SECOM Document

  - Annex A  REST openAPI/swagger (Mattias 10/7)

- S-100 ed 4 (http://s100.iho.int/S100/home/s100-introduction)

- Excel SECOM Tool (command line)

  - openssl, xxd, 7zip commands

- .NET, library (discuss with Mattias and Peter)

- Access to MCP (https://management.maritimecloud.net/#/apply)

- or, Access to Navelink (https://www.navelink.org/enrolment-form/)

- Testdata (RTZ, S-421, S-4xx etc)

  - RTZ_v1.1STM.xsd

  - S421.xsd ?

  - S41x.xsd?

- Document references

- Schedule

APPENDIX SECOM Test Project SMA Report

<placeholder for separate document with Participant report>

ANNEX B Participant (Saab)

Planning

- Implement Testbed A

  - Implement exchange of unclassified signed data (messages such as RTZ, S-421)

  - Implement exchange of classified signed data (messages such as RTZ, S-421)

  - Implement exchange of large classified data (big data as GRIP data, ENC data)

- Implement Testbed B with functionality as VTS (shorecenter)

  - Implement core of Testbed A

  - Implement SECOM Information Service Interface

    - Upload

    - tbd

  - Implement SECOM Data Protection

- Implement Testbed C with functionality as Service provider (chart server, route deviation detection)

  - Implement core of Testbed B

  - Implement SECOM Communication channel security (transport security)

Schedule

See PP for detailed schedule

  - Meetings

  - Work

  - Vacation

  - Milestones

Input

Input will be located on ProjectPlan (https://service.projectplace.com/)

- IEC 63173-2 SECOM Document

  - Annex A  REST openAPI/swagger (Mattias 10/7)

- S-100 ed 4 (http://s100.iho.int/S100/home/s100-introduction)

- Excel SECOM Tool (command line)

  - openssl, xxd, 7zip commands

- .NET, library (discuss with Mattias and Peter)

- Access to MCP (https://management.maritimecloud.net/#/apply)

- or, Access to Navelink (https://www.navelink.org/enrolment-form/)

- Testdata (RTZ, S-421, S-4xx etc)

  - RTZ_v1.1STM.xsd

  - S421.xsd ?

  - S41x.xsd?

- Document references

- Schedule

APPENDIX SECOM Test Project SAAB Report

<placeholder for separate document with Participant report>

ANNEX C Participant (StormGeo)

Planning

- Implement Testbed C

Reference IEC 63173-2 SECOM document

- Implement core of Testbed B

  - Implement exchange of unclassified signed data (messages such as RTZ, S-421)

  - Implement exchange of classified signed data (messages such as RTZ, S-421)

  - Implement SECOM Information Service Interface

    - Upload

    - tbd

  - Implement SECOM Data Protection

  - Implement SECOM Communication channel security (transport security)

Schedule

See PP for detailed schedule

  - Meetings

  - Work

  - Vacation

  - Milestones

Input

Input will be located on ProjectPlan (https://service.projectplace.com/)

- IEC 63173-2 SECOM Document

  - Annex A  REST openAPI/swagger (Mattias 10/7)

- S-100 ed 4 (http://s100.iho.int/S100/home/s100-introduction)

- Excel SECOM Tool (command line)

  - openssl, xxd, 7zip commands

- .NET, library (discuss with Mattias and Peter)

- Access to MCP (https://management.maritimecloud.net/#/apply)

- or, Access to Navelink (https://www.navelink.org/enrolment-form/)

- Testdata (RTZ, S-421, S-4xx etc)

  - RTZ_v1.1STM.xsd

  - S421.xsd ?

  - S41x.xsd?

- Document references

- Schedule

APPENDIX SECOM Test Project StormGeo Report

<placeholder for separate document with Participant report>